Section A — MCQs (5 × 1 = 5 marks)

- b) End-to-end resources are reserved before communication. (Circuit switching.)

- c) L/RL/RL/R. (Packet transmission delay.)

- b) Time Division Multiplexing (TDM).

- c) Packet arrival rate exceeding transmission rate. (Queuing builds; loss if buffer fills.)

- c) Denial of Service (DoS).

Q1 (

Model answer (award up to 5 marks):

- Resource allocation (1–2):

  - Circuit: dedicated path/resources reserved; no sharing during call; predictable performance; idle if unused.

  - Packet: on-demand sharing, no reservation; efficient for bursty traffic; needs congestion/reliability control.

- Efficiency/congestion (1–2):

  - Circuit: wasteful under low utilization, but low delay/jitter under load.

  - Packet: better link utilization overall, but possible queueing delay/loss when arrival>service\text{arrival} > \text{service}arrival>service.

- Use-cases (1):

  - Circuit: constant-bit-rate voice/video with strict latency/jitter.

  - Packet: data apps with burstiness (web, email, most modern apps).

Q2 :

- Given: L = 5,000 bits, R = 2 Mb/s on each of 3 links.

- Per-link transmission delay: tlink=L/R=5000/(2×106)=0.0025t_{\text{link}} = L/R = 5000 / (2 \times 10^6) = 0.0025tlink​=L/R=5000/(2×106)=0.0025 s = 2.5 ms.

- Store-and-forward across 3 links ⇒ total transmission delay
ttotal=3×2.5 ms=7.5 mst_{\text{total}} = 3 \times 2.5 \text{ ms} = \mathbf{7.5\ ms}ttotal​=3×2.5 ms=7.5 ms.

- (Ignoring propagation/processing/queuing, per instruction and consistent with store-and-forward model.) 
Awarding: correct model (2), correct per-link calculation (1), correct total (1), units/assumptions (1).