Computer Organization and Assembly Language (EE2003)

Course Instructor(s):

Mr. Farrukh Bashir, Mr. Shams Farooq Sections: A, B, C ,D ,E ,F , G

| Sessional-I Exam |  |
| --- | --- |
| Total Time (Hrs): | 1 |
| Total Marks: | 60 |
| Total Questions: Date: Sep 24, 2024 | 5 |

___________             ___________                                     _____________________

Roll No                              Course Section                                             Student Signature

Do not write below this line.

Attempt all the questions.

INSTRUCTIONS

- Your final answer should be written with permanent pen, answer will not be acceptable with pencil

- Name all the identifiers in memory

- Consider all the memory starts at address 0x00000100H

- You are supposed to submit your question paper back. NO Answer Book required

|  | Q-1 | Q-2 | Q-3 | Q-4 | Q-5 | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Marks |  |  |  |  |  |  |
| Total | 10 | 10 | 10 | 10 | 20 | 60 |

Question 1 [10 Marks]

Consider the following data declaration and fill in the given memory in hexadecimal (h).

Note: ASCII for ‘A’ = 041H, ‘1’ = 031H

.data b1 byte 10011111b, 16d, 77q, '12AB', 012H,0ABH b2 sbyte "A"+1001b*10q, -1,0FFh, 255 w1 dw 0ABCDH, 'AB','CD' d1 dword 'ABC',0ABCH q1 dq 123456ABCDH,'1234','ABCD'

00; 01; 02; 03; 04; 05; 06; 07; 08; 09; 0A; 0B; 0C; 0D; 0E; 0F. 0100; 9F; 10; 3F; 31; 32; 41; 42; 12; AB; 89; FF; FF; FF; CD; AB; 42. 0110; 41; 44; 43; 43; 42; 41; 00; bc; 0a; 00; 00; 00; cd; ab; 56; 34. 0120; 12; 00; 00; 34; 33; 32; 31; 00; 00; 00; 00; 44; 43; 42; 41; 00. 0130; 00; 00; 00.

Question 2 [5+5 =10 Marks]

Update the given memory and registers after executing the following code.

Note: Consider starting address at 0x00000100H fill memory and registers in HEX

.data w1 dw 5H,6H, 0ABH,8H,9H word 3H bptr LABEL BYTE wptr LABEL WORD dptr LABEL dword qary dq 012345678H count=2 bary1 db 2 dup(count dup(1,2)), 0ABH count=3 bary2 byte 12H, 2 dup(count dup(3))

00; 01; 02; 03; 04; 05; 06; 07; 08; 09; 0A; 0B; 0C; 0D; 0E; 0F. 0100; 05; 00; 06; 00; ab; 00; 08; 00; 09; 00; 03; 00; 78; 56; 34; 12. 0110; 00; 00; 00; 00; 1; 2; 1; 2; 1; 2; 1; 2; AB; 12; 3; 3. 0120; 3; 3; 3; 3. 0130.

mov eax,0 mov al,count mov al, bptr mov bx, [wptr] mov ecx, dptr mov edx, dword ptr[qary+4] mov al, type w1 mov bl, lengthof w1 mov cl, sizeof w1 mov al, type bary1 mov bl, lengthof bary1 mov cl, sizeof bary1 mov al, type bary2 mov bl, lengthof bary2 mov cl, sizeof bary2 count=5 mov al,count; al; 03. al; 78. bx; 5678. ecx; 12345678. edx; 00000000. al; 02. bl; 05. cl; 0A. al; 01. bl; 09. cl; 09. al; 01. bl; 07. cl; 07. al; 05..

Question 3 [10 Marks]

Write an optimized code that copies data from ary1 to ary2 by swapping each byte in the ary1. Your final ary2 should look as shown below

ary2 dw 3412H, 7856H, 0CDABH

NOTE: must use XCHG and LOOP instructions this will produce an optimized code. Do not use unconditional jumps

.data ary1 dw 1234H,5678H,0ABCDH ary2 dw 0, 0, 0

.code mov eax,0 mov ecx, lengthof ary1 mov edi,0 L1: mov ax,ary1[edi] xchg al,ah mov ary1[edi],ax add edi,2 Loop L1

Question 4 [7+3=10 Marks]

Fill memory and update the given memory after executing the following code on data given below.

NOTE: No need to convert characters to ASCII use them as characters. Watch your updated memory carefully.

.data string1 db 'eman srotcurtsni ruoy elcric sunob erocs ot' string2 db $-string1 dup(0) stp dd string1

00; 01; 02; 03; 04; 05; 06; 07; 08; 09; 0A; 0B; 0C; 0D; 0E; 0F. 0100; e; m; a; n; s; r; o; t; c; u; r; t; s; n; I. 0110; r; u; o; y; e; l; c; r; i; c; s; u; n. 0120; o; b; e; r; o; c; s; o; t; t; o; s; c. 0130; o; r; e; b; o; n; u; s; c; i; r; c; l; e. y; o; u; r; i; n; s; t; r; u; c; t; o; r. N; a; m; e.

.code mov eax,0 mov ebx,0 mov eax,offset stp mov esi,[eax] mov ecx, sizeof string1 L1: mov al,[esi+ebx] mov string2[ecx],al inc ebx LOOP L1

Question 5 [  5+5+5+5=20 Marks]

- Update the given flags after executing the following code? No marks for direct answer use rough space mov al,+127 add al,1; OF; SIGN; ZF; AF; PF; CF. 1; 1; 0; 1; 0; 0.. ROUGH WORK: 1111 111 0111 1111 0000 0001 1000 0000.

- Consider the following data declaration and fill in the given memory in hexadecimal (h). NOTE: Highlight spaces create due to align directive and fill them with zero .data var1 db 7,8,9 align 4 var2 dd 012345678H var3 db 2 align 2 var4 dw 5,6 align dword var5 db 7 var6 dw 01234H. 00; 01; 02; 03; 04; 05; 06; 07; 08; 09; 0A; 0B; 0C; 0D; 0E; 0F. 0100; 07; 08; 09; 00; 78; 56; 34; 12; 02; 00; 05; 00; 06; 00; 00; 00. 0110; 07; 34; 12. 0120. 0130..

- Write code that add v1 and v2 and save sum in result for .386 architecture HINT: ADC dest, source ; ADDS destination + source + Carry .data v1 dq 012345678ABCDEFH v2 dq 0123456FFABCDEFH result dq 0. .code mov eax,0 mov eax,dword ptr v1 add eax,dword ptr v2 mov dword ptr result, eax mov eax,dword ptr v1+4 adc eax,dword ptr v2+4 mov dword ptr result+4, eax.

- Update the given flags after executing the following code? What happened to Carry, Overflow and Auxiliary flags. No marks for direct answer use rough space mov bx,1DEFH AND bx,5AA5H; OF; SIGN; ZF; AF; PF; CF. Clear; 0; 0; Clear; 1; Clear.. ROUGH WORK: Clear Carry, Overflow and Aux flags 0001 1101 1110 1111 0101 1010 1010 0101 0001 1000 1010 0101. ROUGH SPACE