Q1: The pipelined RISC-V processor is running the following code snippet. Which registers are being written and which are being read on the fifth cycle? Recall that the pipelined RISC-V processor has a Hazard Unit. You may assume a memory system that returns the result within one cycle.

addi s1, zero, 11 		# s1 = 11

lw s2, 25(s0) 			# s2 = memory[s0+25]

add s3, s3, s4 		# s3 = s3 + s4

or s4, s1, s2 			# s4 = s1 | s2

lw s5, 16(s2) 			# s5 = memory[s2+16]

| Cycle | Fetch | Decode | Execute | Memory | Writeback |
| --- | --- | --- | --- | --- | --- |

Registers Values:

Q2: Using a diagram similar to Figure 7.57, show the forwarding and stalls needed to execute the instructions given below on the pipelined RISC-V processor.

addi s1, zero, 11 			# s1 = 11

lw s2, 25(s1) 				# s2 = memory[36]

lw s5, 16(s2) 				# s5 = memory[s2+16]

add s3, s2, s5 			# s3 = s2 + s5

or s4, s3, t4 				# s4 = s3 | t4

and s2, s3, s4 			# s2 = s3 & s4

Use following diagram notation to draw solution of Q2

Q3: Consider the following instruction sequence in RISC-V assembly language:

L1: 	lw x18, 0(x5) 			# x18 = s2, x5 = t0

sub x18, x18, x19		# x19 = s3

add x5, x18, x19

and x5, x5, x6 			# x6 = t1

addi x7, x5, -1 			# x7 = t2

beq x7, x0, L1

add x28, x5, x18 		# x28 = t3

- Show the timing diagram of the above instruction sequence for a 5-stage RISC-V pipeline (IF, ID, EX, MEM, WB):  without any forwarding but assuming a register read and a write in the same clock cycle “forwards" through the register file.

How many cycles does this code sequence take to execute one iteration?

| Instruction | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 | CC13 | CC14 | CC15 | CC16 | CC17 | CC18 | CC19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Show the pipeline timing assuming full forwarding is enabled and register file supports same-cycle read/write. Show the forwarded values and indicating stalls; i.e,  by circling or drawing clouds as well as labeling the  instructions. Hint: recall that stalling does not use NOP instructions.

How many clock cycles are required for one iteration now?

| Instruction | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 | CC13 | CC14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Using a diagram similar to Figure 7.59, show the flush needed to execute the instructions from Q3 on the pipelined RISC-V processor.

Q4: Consider the delays from Table 7.7 on page 415(book). Now, suppose that the ALU were 20% faster. Would the cycle time of the pipelined RISC-V processor change? What if the ALU were 20% slower? Explain your answers and show your work.