# RISC-V Single-Cycle Datapath

# Question

Consider the following RISC-V program:

- addi x1, x0, 5

- addi x2, x0, 10

- add  x3, x1, x2

- sub  x4, x3, x1

- lw   x5, 0(x3)

- sw   x5, 4(x3)

- beq  x1, x2, LABEL

- lui  x6, 1000

- jal  x7, LABEL

- LABEL:

- add  x8, x1, x1

Fill the datapath control signals table for each instruction, also mention type of the instruction

| Inst | RegWrite | MemRead | MemWrite | MemtoReg | ALUSrc | Branch | ALUOp | ResultSrc | ImmSrc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addi | 1 | 0 | 0 | 0 | 1 | 0 | 10 | 0 | 00 |
| Addi | 1 | 0 | 0 | 0 | 1 | 0 | 10 | 0 | 00 |
| Add | 1 | 0 | 0 | 0 | 0 | 0 | 10 | 0 | XX |
| Sub | 1 | 0 | 0 | 0 | 0 | 0 | 10 | 0 | XX |
| Lw | 1 | 1 | 0 | 1 | 1 | 0 | 00 | 1 | 00 |
| Sw | 0 | 0 | 1 | X | 1 | 0 | 00 | X | 01 |
| Beq | 0 | 0 | 0 | X | 0 | 1 | 01 | X | 10 |
| Lui | 1 | 0 | 0 | 11 | X | 0 | XX | 11 | 100 |
| Jal | 1 | 0 | 0 | 10 | X | 0 | XX | 10 | 11 |
| Add | 1 | 0 | 0 | 00 | 0 | 0 | 10 | 00 | XX |