# RISC-V Single-Cycle Datapath (Section-F)

# Question

Consider the following RISC-V program:

- addi x10, x0, 15

- addi x11, x0, 25

- sub  x12, x11, x10

- and  x13, x10, x11

- lw   x14, 8(x12)

- sw   x14, 12(x12)

- bne  x10, x11, TARGET

- lui x15, 2000

- jal  x16, TARGET

- TARGET:

- or   x17, x10, x11

Fill the datapath control signals table for each instruction, also mention type of the instruction

| Inst | Type | RegWrite | MemRead | MemWrite | MemtoReg | ALUSrc | Branch | ALUOp | ResultSrc | ImmSrc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| addi x10, x0, 15 | I | 1 | 0 | 0 | 0 | 1 | 0 | 10 | 0 | 00 |
| addi x11, x0, 25 | I | 1 | 0 | 0 | 0 | 1 | 0 | 10 | 0 | 00 |
| sub x12, x11, x10 | R | 1 | 0 | 0 | 0 | 0 | 0 | 10 | 0 | XX |
| and x13, x10, x11 | R | 1 | 0 | 0 | 0 | 0 | 0 | 10 | 0 | XX |
| lw x14, 8(x12) | I | 1 | 1 | 0 | 1 | 1 | 0 | 00 | 1 | 00 |
| sw x14, 12(x12) | S | 0 | 0 | 1 | X | 1 | 0 | 00 | X | 01 |
| bne x10, x11, TARGET | B | 0 | 0 | 0 | X | 0 | 1 | 01 | X | 10 |
| lui x15, 2000 | U | 1 | 0 | 0 | 0 | 1 | 0 | 00 | 00 | 11 |
| jal x16, TARGET | J | 1 | 0 | 0 | X | 1 | 0 | 01 | 10 | 11 |
| or x17, x10, x11 | R | 1 | 0 | 0 | 0 | 0 | 0 | 10 | 0 | XX |